Iowa State University

Electrical And Computer Engineering

EE 230

LAB 8 REPORT

April 21, 2026

Sayon Saha

Aaron Kammlah

Sayon Saha

# Introduction

In this lab, we started working on BJT transistors. At first, we took 4 different transistors, including the PN2222 and PN2907 transistors, and used them in the parameter analyzer to find their BF values and get the iC-VCE traces. We compared the value with our measured one and recorded them for use in the next experiments. Then we built 3 different circuits with the PN2222 and one with the PN2907, and measured the base, collector and emitter current along with the base-emitter voltage and collector-emitter voltage. Then we compared those values with our calculated values. Then we made a BJT switch with the PN2222 and collected data across 25 different Vi and visualized the behavior through graphs. Finally, we built an amplifier circuit with a PN2222 to light up a LED and observed how adding the BJT improved the performance, and then we proceeded to build an amplifier with a BJT.

# A	Quick measure of βF and iC – vCE characteristics with the parameter analyzer

| Circuits | Calculated Values |
| --- | --- |
|  | Collector Current, iC = 9.2 mA Base Current, iB = 0.043 mA Forward current gain, BF = 213.95 BF = ic/ib = 213.95 |
|  | Collector Current, iC = 7 mA Base Current, iB = 0.043 mA Forward current gain, BF = 162.79 BF = ic/ib = 162.79 |

Figure 1: iC –vCE curves for PN2222

Figure 2: iC –vCE curves for PN2906

# Simple BJT Circuits

|  | iB | iC | iE | vBE | vCE |
| --- | --- | --- | --- | --- | --- |
| Measured | 0.0055 mA | 1.27 mA | 1.82 mA | 0.63 V | 5.28 V |
| Calculated | 0.00576 mA | 1.232 mA | 1.238 mA | 0.7 V | 5,43 V |

|  | iB | iC | iE | vBE | vCE |
| --- | --- | --- | --- | --- | --- |
| Measured | 0.018 mA | 4.082 mA | 4.16 mA | 0.66 V | 5.83 V |
| Calculated | 0.0191 mA | 4.090 mA | 4.109 mA | 0.7 V | 5.89 V |

|  | iB | iC | iE | vBE | vCE |
| --- | --- | --- | --- | --- | --- |
| Measured | 1.68 mA | 2.6 mA | 4.28 mA | 0.726 | 0.096 V |
| Calculated | 1.80 mA | 2.04 mA | 4.3 mA | 0.7 V | 0.2 V |

|  | iB | iC | iE | vBE | vCE |
| --- | --- | --- | --- | --- | --- |
| Measured | 0.016 mA | 2.7 mA | 2.7 mA | -0.66 V | -4.59 V |
| Calculated | 0.0163 mA | 2.65 mA | 2.67 mA | -0.7 V | -4.68 V |

# C	BJT as a switch

Figure 4: Transfer Characteristics, plot of vo vs vi

Figure 5: Plot of ic vs vi

Figure 6: Plot of PR vs vi

Figure 7: Plot of Pnpn vs vi

Figure 8: Plot of PR vs Pnpn

Measured Output Voltage, vo  = 7.57 V

Measured Output Current, io  = 0.757 mA

Measured Output Voltage, vo  = 9.95 V

Measured Output Current, io  = 72.7 mA

In part a, the voltage and current are lower than ideal because the op-amp is directly driving the LED load. The amp cannot source a large output current. As such when the LED and resistor demand current, the output voltage drops below the 10 V supply rail. But in part b, the transistor output stage fixes this problem by letting the op amp supply only a small base current while the transistor supplies the larger LED current.

Figure 9: Oscilloscope Trace

Figure 10: Oscilloscope Trace

D	BJT as an amplifier

Gain = 6.72

# Conclusion

In this lab, we didn’t face any major problems. All the experiments were completed without any issues, and all our measured values were very close to our calculated values, they were all within 5% of our calculated values. The plots we derived from excel also matched our expectations. The lab helped us visualize the theory we learned about BJTs.